Tập trung vào dữ liệu điện

| Bảng | Vai trò |
| --- | --- |
| data_iot_convert | Bảng fact trung tâm, chứa log trạng thái vận hành theo thời gian |
| data_machine | Dimension máy, giúp biết tên máy, model, nhóm máy, mã IoT của máy |
| data_machine_status | Dimension trạng thái, giải nghĩa status_id |
| machine_location_his | Lịch sử vị trí máy theo thời gian |
| data_location | Tên khu vực/xưởng/vị trí |

status_id không nên là nhãn chính cho bài toán bất thường?

Vì “có lỗi” không đồng nghĩa với “bất thường”, và “bất thường” không phải lúc nào cũng đã có lỗi.

Ví dụ:

Trường hợp 1: Có lỗi, nhưng không phải bất thường theo nghĩa vận hành

Máy đang sửa chữa, status 6 hoặc 7:

6 = Sửa chữa không tải

7 = Sửa chữa có tải

Đây là “có lỗi”, nhưng nếu máy đang trong lịch bảo trì/sửa chữa thì nó có thể là trạng thái hợp lệ, không phải bất thường cần cảnh báo mới.

Trường hợp 2: Không lỗi, nhưng vẫn bất thường

Ví dụ status vẫn là:

2 = Chạy sản xuất không tải

3 = Chạy sản xuất có tải

Nhưng máy có hành vi lạ:

status 2 kéo dài 10 tiếng

status 3 có kWh_delta = 0

máy chuyển 2 ↔ 3 hàng nghìn lần trong 1 giờ

máy thường chạy ban ngày nhưng đêm nay chạy liên tục

máy tiêu thụ kWh cao gấp 5 lần lịch sử

Các trường hợp này không nhất thiết có status_id lỗi, nhưng vẫn là bất thường vận hành.

Lớp 1: Behavior Anomaly Detection

→ Hiện tại máy có đang vận hành lệch khỏi nền lịch sử của chính nó không?

Lớp 2: Fault Confidence / Fault Judgment

→ Sự lệch đó có đáng tin không, có khả năng là lỗi/sự cố/bảo trì bất thường không?

| Bảng | Cột chọn | Vai trò | Hướng xử lý |
| --- | --- | --- | --- |
| data_iot_convert | id | ID sự kiện | Không đưa vào model, chỉ dùng trace ngược lại event gốc |
| data_iot_convert | machine_id | Khóa học lịch sử từng máy | Bắt buộc. Model học baseline riêng theo từng machine_id |
| data_iot_convert | status_id | Feature trạng thái | Join sang data_machine_status.id; tạo phân bố trạng thái theo giờ/ngày |
| data_iot_convert | status_time_start | Mốc thời gian chính | Tạo window_start, hour_of_day, day_of_week, operation_date |
| data_iot_convert | status_time_end | Mốc kết thúc trạng thái | Tính duration_sec; xử lý null bằng cờ is_open_event |
| data_iot_convert | status_kwh_start | Điện năng đầu event | Tính kwh_delta; do thiếu khoảng 55.77% nên không được coi là feature bắt buộc tuyệt đối |
| data_iot_convert | status_kwh_end | Điện năng cuối event | Tính kwh_delta = end - start |
| data_iot_convert | note | Feature giải nghĩa trạng thái | Dùng kiểm tra/backup với data_machine_status.note; có thể tách ON/OFF, Có lỗi, Có bảo trì, Dòng=0, Dòng>0 |
| data_iot_convert | is_deleted | Cột lọc | Chỉ lấy is_deleted = 0 |
| data_machine_status | id | Khóa join | Join với data_iot_convert.status_id; quan hệ này coverage 1.0 và note consistency đúng trên sample |
| data_machine_status | status_name | Metadata giải thích | Dùng cho báo cáo lý do bất thường |
| data_machine_status | iottag_part_signal | Feature loại tín hiệu | Ví dụ RunPdNoLoad, RunPdOnLoad, PowerOff |
| data_machine_status | type | Feature trạng thái tổng | Tạo on_duration_sec, off_duration_sec, info_duration_sec |
| data_machine_status | note | Feature semantic chính | Tách is_loaded, is_no_load, has_error_token, has_maintenance_token |
| data_machine_status | group_id | Feature phụ | Dùng thận trọng vì thiếu bảng dimension giải nghĩa group_id |
| data_machine | id | Khóa join | Join với data_iot_convert.machine_id; coverage 1.0 |
| data_machine | machine_name | Metadata | Không encode text thô ở phase đầu; dùng để hiển thị kết quả |
| data_machine | machine_group_id | Feature ngữ cảnh máy | Dùng nếu train model chung nhiều máy; nếu train riêng từng máy thì chỉ dùng phụ |
| data_machine | machine_model | Feature ngữ cảnh | Encode categorical hoặc dùng metadata giải thích |
| data_machine | machine_branch_id | Feature ngữ cảnh | Dùng phụ; đang thiếu dimension giải nghĩa |
| data_machine | year_of_production | Feature tuổi máy | Xử lý 0 là missing |
| data_machine | machine_supplier | Feature nhà cung cấp | Encode categorical |
| data_machine | iottag_part_machine | Mapping IoT | Dùng kiểm tra mapping/tag, không phải feature chính |
| machine_location_his | machine_id | Khóa join vị trí | Join theo machine_id + thời gian |
| machine_location_his | location_id | Feature vị trí | Gắn máy vào khu vực vận hành |
| machine_location_his | start_time | Hiệu lực vị trí | Join điều kiện status_time_start >= start_time |
| machine_location_his | end_time | Hiệu lực vị trí | Nếu null nghĩa là vị trí còn hiệu lực |
| machine_location_his | is_deleted | Cột lọc | Chỉ lấy is_deleted = 0 |
| data_location | id | Khóa join | Join với machine_location_his.location_id; coverage 1.0 |
| data_location | location_code | Feature/metadata vị trí | Ví dụ CNC Thanh, CNC Mã |
| data_location | location_name | Metadata | Dùng giải thích kết quả |
| data_location | location_parent_id | Feature phân cấp | Dùng phụ; thiếu dimension giải nghĩa |
| data_location | location_level_id | Feature cấp vị trí | Dùng phụ; thiếu dimension giải nghĩa |
| data_location | is_deleted | Cột lọc | Chỉ lấy is_deleted = 0 |

| Bảng / nguồn | Cột chọn | Vai trò | Hướng xử lý |
| --- | --- | --- | --- |
| Output lớp 1 | behavior_anomaly_score | Feature chính | Điểm lệch hành vi |
| Output lớp 1 | is_behavior_anomaly | Feature/cổng kích hoạt | Chỉ đánh giá lỗi sâu khi lớp 1 báo lệch |
| Output lớp 1 | behavior_reason | Feature giải thích | Dùng trong báo cáo cuối |
| data_iot_convert | status_id | Bằng chứng lỗi/bảo trì | Tạo các nhãn phụ từ mapping status |
| data_iot_convert | note | Bằng chứng lỗi/bảo trì | Tách text: Có lỗi, Có bảo trì, OFF, Dòng>0, Dòng=0 |
| data_iot_convert | error_code | Bằng chứng lỗi | Hiện báo cáo cho thấy missing 100%, nên giữ schema nhưng chưa dùng được |
| data_iot_convert | status_time_start, status_time_end | Thời lượng bằng chứng | Tính error_duration_sec, maintenance_duration_sec, repair_duration_sec |
| data_iot_convert | status_kwh_start, status_kwh_end | Bằng chứng năng lượng | Tạo abnormal_kwh_flag, ví dụ có tải nhưng kWh không đổi |
| data_machine_status | id | Mapping trạng thái | Join với status_id |
| data_machine_status | status_name | Giải thích lỗi | Ví dụ “Sửa chữa không tải”, “Tắt máy có sự cố” |
| data_machine_status | iottag_part_signal | Phân loại tín hiệu | Tách nhóm RunReNoLoad, RunReOnLoad, PowerOffErr |
| data_machine_status | type | Bằng chứng ON/OFF/INFO | Phân biệt lỗi khi máy ON và lỗi khi máy OFF |
| data_machine_status | note | Nguồn tạo label phụ | Tạo known_fault_status, known_maintenance_status, known_repair_status |
| data_machine_status | is_show | Lọc tín hiệu chính/phụ | Các status 11-14 là INFO/tag bổ sung, không nên coi giống trạng thái vận hành chính |
| data_machine_status | group_id | Feature phụ | Dùng sau nếu có bảng giải nghĩa group |
| data_machine | machine_group_id | Ngữ cảnh tin cậy | Một số nhóm máy có nền vận hành/lỗi khác nhau |
| data_machine | machine_model | Ngữ cảnh tin cậy | Cùng bất thường nhưng model máy khác nhau có mức rủi ro khác |
| data_machine | machine_supplier | Ngữ cảnh tin cậy | Có thể dùng khi phân tích theo hãng |
| machine_location_his | location_id | Ngữ cảnh khu vực | Nếu nhiều máy cùng khu lệch, có thể là vấn đề khu vực, không chỉ lỗi riêng máy |
| data_location | location_code, location_name | Giải thích khu vực | Dùng trong báo cáo phán đoán |
| data_location | location_parent_id, location_level_id | Ngữ cảnh phân cấp | Dùng phụ, chưa phải feature mạnh |

event thô có độ dài không đều. Có event kéo dài vài giây, có event kéo dài vài phút, có event kéo dài hàng giờ. AI tabular như Isolation Forest, XGBoost, Random Forest, AutoEncoder dạng bảng sẽ cần mỗi dòng có cùng ý nghĩa. Nếu mỗi dòng là một event rời rạc thì model chỉ thấy “một trạng thái”, không thấy được nhịp vận hành tổng thể.

Dùng cửa sổ trượt theo event theo cụm máy và theo chuỗi thời gian

Query tạo bảng lớp 1: (Đã theo chuỗi thời gian và gom cụm máy chưa?)

IF OBJECT_ID('dbo.ai_l1_operation_event_sequence', 'U') IS NOT NULL

DROP TABLE dbo.ai_l1_operation_event_sequence;

GO

WITH base_event AS (

SELECT

iot.id AS event_id,

iot.machine_id,

iot.status_id,

iot.status_time_start,

iot.status_time_end,

iot.status_kwh_start,

iot.status_kwh_end,

iot.note AS event_note,

iot.error_code,

ms.status_name,

ms.iottag_part_signal,

ms.[type] AS status_type,

ms.is_show AS status_is_show,

ms.note AS status_note,

ms.group_id AS status_group_id,

m.machine_name,

m.machine_group_id,

m.machine_model,

m.machine_branch_id,

m.machine_call_name,

m.machine_serial_no,

m.year_of_production,

m.machine_supplier,

m.iottag_part_machine,

mlh.location_id,

loc.location_code,

loc.location_name,

loc.location_parent_id,

loc.location_level_id

FROM dbo.data_iot_convert iot

LEFT JOIN dbo.data_machine_status ms

ON iot.status_id = ms.id

AND ISNULL(ms.is_deleted, 0) = 0

LEFT JOIN dbo.data_machine m

ON iot.machine_id = m.id

OUTER APPLY (

SELECT TOP 1

h.location_id,

h.start_time,

h.end_time

FROM dbo.machine_location_his h

WHERE h.machine_id = iot.machine_id

AND ISNULL(h.is_deleted, 0) = 0

AND iot.status_time_start >= h.start_time

AND (

h.end_time IS NULL

OR iot.status_time_start < h.end_time

)

ORDER BY h.start_time DESC

) mlh

LEFT JOIN dbo.data_location loc

ON mlh.location_id = loc.id

AND ISNULL(loc.is_deleted, 0) = 0

WHERE ISNULL(iot.is_deleted, 0) = 0

AND iot.machine_id IS NOT NULL

AND iot.status_id IS NOT NULL

AND iot.status_time_start IS NOT NULL

),

ordered_event AS (

SELECT

b.*,

LAG(b.status_id) OVER (

PARTITION BY b.machine_id

ORDER BY b.status_time_start, b.event_id

) AS prev_status_id,

LEAD(b.status_id) OVER (

PARTITION BY b.machine_id

ORDER BY b.status_time_start, b.event_id

) AS next_status_id,

LAG(b.status_time_end) OVER (

PARTITION BY b.machine_id

ORDER BY b.status_time_start, b.event_id

) AS prev_status_time_end,

LEAD(b.status_time_start) OVER (

PARTITION BY b.machine_id

ORDER BY b.status_time_start, b.event_id

) AS next_status_time_start

FROM base_event b

),

clean_event AS (

SELECT

o.*,

CASE

WHEN o.status_time_end IS NOT NULL THEN o.status_time_end

WHEN o.next_status_time_start IS NOT NULL THEN o.next_status_time_start

ELSE NULL

END AS effective_status_time_end,

CASE

WHEN o.status_time_end IS NULL THEN 1 ELSE 0

END AS is_open_event,

CASE

WHEN o.status_time_end IS NULL

AND o.next_status_time_start IS NOT NULL THEN 1

ELSE 0

END AS end_time_imputed_flag,

DATEDIFF(

SECOND,

o.status_time_start,

CASE

WHEN o.status_time_end IS NOT NULL THEN o.status_time_end

WHEN o.next_status_time_start IS NOT NULL THEN o.next_status_time_start

ELSE o.status_time_start

END

) AS duration_sec,

CASE

WHEN o.prev_status_time_end IS NULL THEN NULL

ELSE DATEDIFF(SECOND, o.prev_status_time_end, o.status_time_start)

END AS gap_from_prev_sec,

CASE

WHEN o.prev_status_time_end IS NOT NULL

AND o.status_time_start < o.prev_status_time_end

THEN DATEDIFF(SECOND, o.status_time_start, o.prev_status_time_end)

ELSE 0

END AS overlap_sec,

CASE

WHEN o.status_kwh_start IS NOT NULL

AND o.status_kwh_end IS NOT NULL

THEN o.status_kwh_end - o.status_kwh_start

ELSE NULL

END AS kwh_delta,

CASE

WHEN o.status_kwh_start IS NULL

OR o.status_kwh_end IS NULL

THEN 1 ELSE 0

END AS kwh_missing_flag,

CASE WHEN o.status_type = 'ON' THEN 1 ELSE 0 END AS is_on,

CASE WHEN o.status_type = 'OFF' THEN 1 ELSE 0 END AS is_off,

CASE WHEN o.status_type = 'INFO' THEN 1 ELSE 0 END AS is_info,

CASE

WHEN ISNULL(o.status_note, o.event_note) LIKE N'%Dòng>0%' THEN 1 ELSE 0

END AS is_loaded,

CASE

WHEN ISNULL(o.status_note, o.event_note) LIKE N'%Dòng=0%' THEN 1 ELSE 0

END AS is_no_load,

CASE

WHEN ISNULL(o.status_note, o.event_note) LIKE N'%Có lỗi%' THEN 1 ELSE 0

END AS has_error_token,

CASE

WHEN ISNULL(o.status_note, o.event_note) LIKE N'%Có bảo trì%' THEN 1 ELSE 0

END AS has_maintenance_token,

DATEPART(HOUR, o.status_time_start) AS hour_of_day,

DATEPART(WEEKDAY, o.status_time_start) AS day_of_week,

CAST(o.status_time_start AS date) AS operation_date

FROM ordered_event o

),

flagged_event AS (

SELECT

c.*,

CASE

WHEN c.duration_sec <= 0 THEN 1

WHEN c.duration_sec > 24 * 3600 THEN 1

ELSE 0

END AS is_bad_duration,

CASE

WHEN c.gap_from_prev_sec IS NULL THEN 0

WHEN c.gap_from_prev_sec > 60 * 60 THEN 1

ELSE 0

END AS is_big_gap,

CASE

WHEN c.gap_from_prev_sec IS NULL THEN 0

WHEN c.gap_from_prev_sec > 5 * 60 THEN 1

ELSE 0

END AS is_gap,

CASE

WHEN c.overlap_sec > 0 THEN 1 ELSE 0

END AS is_overlap,

CASE

WHEN c.is_loaded = 1

AND c.status_kwh_start IS NOT NULL

AND c.status_kwh_end IS NOT NULL

AND c.status_kwh_end - c.status_kwh_start = 0

THEN 1 ELSE 0

END AS loaded_zero_kwh_flag,

CASE

WHEN c.is_loaded = 1

AND (c.status_kwh_start IS NULL OR c.status_kwh_end IS NULL)

THEN 1 ELSE 0

END AS loaded_without_kwh_flag

FROM clean_event c

),

segmented_event AS (

SELECT

f.*,

SUM(

CASE

WHEN f.prev_status_id IS NULL THEN 1

WHEN f.is_big_gap = 1 THEN 1

WHEN f.is_bad_duration = 1 THEN 1

ELSE 0

END

) OVER (

PARTITION BY f.machine_id

ORDER BY f.status_time_start, f.event_id

ROWS UNBOUNDED PRECEDING

) AS sequence_segment_id

FROM flagged_event f

)

SELECT

ROW_NUMBER() OVER (

PARTITION BY machine_id, sequence_segment_id

ORDER BY status_time_start, event_id

) AS event_order_in_segment,

event_id,

machine_id,

status_id,

prev_status_id,

next_status_id,

status_time_start,

status_time_end,

effective_status_time_end,

duration_sec,

prev_status_time_end,

gap_from_prev_sec,

overlap_sec,

status_kwh_start,

status_kwh_end,

kwh_delta,

kwh_missing_flag,

loaded_zero_kwh_flag,

loaded_without_kwh_flag,

event_note,

error_code,

status_name,

iottag_part_signal,

status_type,

status_is_show,

status_note,

status_group_id,

is_on,

is_off,

is_info,

is_loaded,

is_no_load,

has_error_token,

has_maintenance_token,

is_open_event,

end_time_imputed_flag,

is_bad_duration,

is_gap,

is_big_gap,

is_overlap,

sequence_segment_id,

machine_name,

machine_group_id,

machine_model,

machine_branch_id,

machine_call_name,

machine_serial_no,

CASE WHEN year_of_production = 0 THEN NULL ELSE year_of_production END AS year_of_production,

machine_supplier,

iottag_part_machine,

location_id,

location_code,

location_name,

location_parent_id,

location_level_id,

hour_of_day,

day_of_week,

operation_date,

CAST(NULL AS float) AS behavior_anomaly_score,

CAST(NULL AS bit) AS is_behavior_anomaly,

CAST(NULL AS nvarchar(500)) AS behavior_reason

INTO dbo.ai_l1_operation_event_sequence

FROM segmented_event;

GO

CREATE INDEX IX_ai_l1_machine_time

ON dbo.ai_l1_operation_event_sequence(machine_id, status_time_start, event_id);

CREATE INDEX IX_ai_l1_segment

ON dbo.ai_l1_operation_event_sequence(machine_id, sequence_segment_id, event_order_in_segment);

GO

Query tạo bảng lớp 2:

IF OBJECT_ID('dbo.ai_l2_fault_confidence_event', 'U') IS NOT NULL

DROP TABLE dbo.ai_l2_fault_confidence_event;

GO

WITH l1 AS (

SELECT

*,

TRY_CONVERT(int, status_id) AS status_id_int

FROM dbo.ai_l1_operation_event_sequence

)

SELECT

-- =========================================================

-- 1. Khóa liên kết 1-1 với lớp 1

-- =========================================================

l1.event_id,

l1.machine_id,

l1.sequence_segment_id,

l1.event_order_in_segment,

-- =========================================================

-- 2. Thời gian và thứ tự chuỗi

-- =========================================================

l1.status_time_start,

l1.status_time_end,

l1.effective_status_time_end,

l1.prev_status_time_end,

l1.prev_status_id,

l1.next_status_id,

-- =========================================================

-- 3. Trạng thái vận hành hiện tại

-- =========================================================

l1.status_id,

l1.status_name,

l1.iottag_part_signal,

l1.status_type,

l1.status_is_show,

l1.status_note,

l1.status_group_id,

l1.is_on,

l1.is_off,

l1.is_info,

l1.is_loaded,

l1.is_no_load,

l1.has_error_token,

l1.has_maintenance_token,

-- =========================================================

-- 4. Nhãn phụ / bằng chứng lỗi từ status

-- =========================================================

CASE

WHEN l1.status_id_int IN (6, 7, 9, 10)

OR l1.has_error_token = 1

THEN 1 ELSE 0

END AS known_fault_status,

CASE

WHEN l1.status_id_int IN (4, 5, 6, 7, 10)

OR l1.has_maintenance_token = 1

THEN 1 ELSE 0

END AS known_maintenance_status,

CASE

WHEN l1.status_id_int IN (6, 7)

THEN 1 ELSE 0

END AS known_repair_status,

CASE

WHEN l1.status_id_int IN (9, 10)

THEN 1 ELSE 0

END AS off_with_fault_status,

CASE

WHEN l1.status_id_int IN (11, 12, 13, 14)

OR l1.status_type = 'INFO'

THEN 1 ELSE 0

END AS info_status,

-- =========================================================

-- 5. Thời lượng, gap, overlap

-- =========================================================

l1.duration_sec,

l1.gap_from_prev_sec,

l1.overlap_sec,

l1.is_open_event,

l1.end_time_imputed_flag,

l1.is_bad_duration,

l1.is_gap,

l1.is_big_gap,

l1.is_overlap,

-- =========================================================

-- 6. KWh và chất lượng đo điện

-- =========================================================

l1.status_kwh_start,

l1.status_kwh_end,

l1.kwh_delta,

l1.kwh_missing_flag,

l1.loaded_zero_kwh_flag,

l1.loaded_without_kwh_flag,

CASE

WHEN l1.is_loaded = 1

AND l1.kwh_missing_flag = 1

THEN 1 ELSE 0

END AS loaded_kwh_unreliable_flag,

CASE

WHEN l1.is_loaded = 1

AND l1.loaded_zero_kwh_flag = 1

THEN 1 ELSE 0

END AS loaded_but_no_energy_increase_flag,

-- =========================================================

-- 7. Error code

-- =========================================================

l1.error_code,

CASE

WHEN l1.error_code IS NOT NULL

AND LTRIM(RTRIM(l1.error_code)) <> ''

THEN 1 ELSE 0

END AS has_error_code,

-- =========================================================

-- 8. Ngữ cảnh máy

-- =========================================================

l1.machine_name,

l1.machine_group_id,

l1.machine_model,

l1.machine_branch_id,

l1.machine_call_name,

l1.machine_serial_no,

l1.year_of_production,

l1.machine_supplier,

l1.iottag_part_machine,

-- =========================================================

-- 9. Ngữ cảnh vị trí

-- =========================================================

l1.location_id,

l1.location_code,

l1.location_name,

l1.location_parent_id,

l1.location_level_id,

-- =========================================================

-- 10. Ngữ cảnh thời gian

-- =========================================================

l1.hour_of_day,

l1.day_of_week,

l1.operation_date,

-- =========================================================

-- 11. Output lớp 1, sau này model lớp 1 sẽ update

-- =========================================================

l1.behavior_anomaly_score,

l1.is_behavior_anomaly,

l1.behavior_reason,

-- =========================================================

-- 12. Data quality score sơ bộ cho lớp 2

-- =========================================================

CASE

WHEN l1.is_bad_duration = 1

OR l1.is_overlap = 1

OR l1.is_big_gap = 1

OR l1.is_open_event = 1

THEN 1 ELSE 0

END AS data_quality_issue_flag,

CASE

WHEN l1.is_bad_duration = 1 THEN N'BAD_DURATION'

WHEN l1.is_overlap = 1 THEN N'OVERLAP_EVENT'

WHEN l1.is_big_gap = 1 THEN N'BIG_GAP'

WHEN l1.is_open_event = 1 THEN N'OPEN_EVENT'

WHEN l1.kwh_missing_flag = 1 THEN N'KWH_MISSING'

ELSE N'OK'

END AS data_quality_reason,

-- =========================================================

-- 13. Output lớp 2, sau này rule/model sẽ ghi vào

-- =========================================================

CAST(NULL AS float) AS fault_confidence_score,

CAST(NULL AS nvarchar(100)) AS fault_judgment,

CAST(NULL AS nvarchar(100)) AS fault_type_guess,

CAST(NULL AS nvarchar(100)) AS action_level,

CAST(NULL AS nvarchar(1000)) AS final_reason

INTO dbo.ai_l2_fault_confidence_event

FROM l1;

GO

CREATE UNIQUE INDEX IX_ai_l2_fault_event_id

ON dbo.ai_l2_fault_confidence_event(event_id);

GO

CREATE INDEX IX_ai_l2_fault_machine_sequence

ON dbo.ai_l2_fault_confidence_event(

machine_id,

sequence_segment_id,

event_order_in_segment

);

GO

CREATE INDEX IX_ai_l2_fault_machine_time

ON dbo.ai_l2_fault_confidence_event(

machine_id,

status_time_start

);

GO

Kiểm tra 1-1 với lớp 1: l1_rows = l2_rows

SELECT COUNT(*) AS l1_rows

FROM dbo.ai_l1_operation_event_sequence;

SELECT COUNT(*) AS l2_rows

FROM dbo.ai_l2_fault_confidence_event;

Kiểm tra có trùng event_id với lớp 1 không (kỳ vọng không trùng???)

SELECT

event_id,

COUNT(*) AS cnt

FROM dbo.ai_l2_fault_confidence_event

GROUP BY event_id

HAVING COUNT(*) > 1;

Khuyến nghị nên thêm 3 cờ này:

CASE WHEN ms.id IS NULL THEN 1 ELSE 0 END AS status_missing_flag,

CASE WHEN m.id IS NULL THEN 1 ELSE 0 END AS machine_missing_flag,

CASE WHEN mlh.location_id IS NULL THEN 1 ELSE 0 END AS location_missing_flag

Vì sau này nếu model hoặc báo cáo thấy dữ liệu bất thường, ta biết đó là bất thường vận hành hay bất thường do thiếu master data.

Sửa join máy: